THE WORKS FREIGHT ADVISORY REPORT 8 × Volvo Prime Mover + Fuel Tanker ¦ Melbourne → Jeddah ¦ RoRo 27 July 2026

| Field | Detail |
| --- | --- |
| Prepared by | Soren (Global Freight Specialist) + Remy (Compliance Specialist) |
| Authorised by | MEO — Orchestrator, The Works |
| Date | 27 July 2026 |
| Cargo | 8 × Volvo prime mover + fuel tanker road train combination |
| Dimensions | 18 m L × 3 m W × 2.8 m H per unit |
| GVM | 20,000 kg per unit |
| Total lane metres | 144 LM (length basis); 216 LM if 3 m width factor applies — confirm with carrier |
| DG status | Degassed — Gas-Free Certificates held; UN 1202 Class 3 PG III (diesel fuel) |
| SASO | Confirmed held by owner for Jeddah |
| Destination | Jeddah Islamic Port (King Abdulaziz Port), Saudi Arabia |
| Priority | Cost minimisation |

PART 1 — OPERATING ENVIRONMENT: DUAL CHOKEPOINT CRISIS

⚠ CRITICAL CONTEXT — READ BEFORE ROUTING DECISIONS Both major chokepoints on the standard Australia–Middle East freight corridor are simultaneously disrupted as at 27 July 2026. This is not a normal market condition. All routing decisions in this report are assessed against this backdrop.

1.1  Red Sea / Bab el-Mandeb — Active Houthi Maritime Embargo

On 20 July 2026, Houthi forces declared a maritime embargo specifically targeting Saudi-bound commercial vessels, stating ships calling at Saudi terminals face targeting anywhere within their operational reach. On 22 July 2026, the Houthis claimed attacks on two Saudi oil tankers.

War risk insurance for southern Red Sea voyages has surged from approximately 0.3% of vessel value pre-announcement to 0.75–1%+ as of 21–23 July 2026, with some Saudi-linked cargo quoted as high as 3%.

Source: MARAD Advisory 2026-006; Insurance Journal 21–23 Jul 2026; Al Jazeera 22 Jul 2026 — observed 27 Jul 2026.

1.2  Strait of Hormuz — Day 148 of Closure

On 28 February 2026, the United States and Israel launched military operations against Iran. Iran declared the Strait of Hormuz closed to commercial traffic. As at 27 July 2026, this is Day 148 of the crisis. Vessel traffic is down approximately 90% year-on-year — approximately 10 ships/day against a pre-crisis baseline of ~88/day. Approximately 70% of remaining transits run dark (AIS disabled).

Jebel Ali (Dubai) and Dammam (Saudi Arabia, Arabian Gulf) are effectively inaccessible via normal Hormuz routing.

Source: straits.live live tracker; Windward Daily Intelligence; Lloyd's List Intelligence 21 Jul 2026; CNBC 21 Jul 2026 — observed 27 Jul 2026.

1.3  Implication for This Shipment

The standard Melbourne → Indian Ocean → Hormuz → Jebel Ali/Dammam path is effectively blocked. The Melbourne → Indian Ocean → Bab el-Mandeb → Jeddah path carries active war risk targeting Saudi-bound vessels.

The viable alternative corridor is: Indian Ocean → Gulf of Oman → UAE east coast ports (Khorfakkan or Fujairah). These ports are accessible WITHOUT transiting Hormuz, and without Red Sea exposure. All three routing scenarios in this report are assessed against this reality.

PART 2 — ROUTING SCENARIOS

2.1  Scenario A — Melbourne → Red Sea → Jeddah Islamic Port (ORIGINAL)

⚠ NOT RECOMMENDED — Active Houthi maritime embargo on Saudi-bound vessels Do not book Scenario A without board-level risk acceptance from the owner. WWL has moved all vessels to Cape of Good Hope diversion. War risk surcharge is live and escalating.

Route: Melbourne → Singapore or Port Klang (transhipment) → Indian Ocean → Gulf of Aden → Bab el-Mandeb → Red Sea → Jeddah Islamic Port.

Carrier positions as at 27 July 2026:

WWL: All vessels diverted via Cape of Good Hope. Can reach Jeddah from north (Cape → Atlantic → Mediterranean → Suez Canal → Red Sea northward to Jeddah), avoiding Bab el-Mandeb. Transit time premium: +14–20 days vs. direct. Cost premium: ~25–40%.

Höegh: Historically continued Red Sea transit. Current position post-20 Jul embargo not confirmed.

NYK: Schedule includes Jeddah. Current routing post-embargo not confirmed in this research.

K-Line / MOL: Routing position not confirmed.

| Parameter | Detail |
| --- | --- |
| Sea transit (direct via Bab el-Mandeb) | 35–42 days Melbourne to Jeddah (inc. Singapore transhipment 5–10 days) |
| Sea transit (WWL Cape/Suez approach) | 50–60 days Melbourne to Jeddah |
| War risk surcharge (sea leg) | 0.75–3% of cargo declared value — active as at 23 Jul 2026 |
| War risk surcharge (cost impact) | USD 8,000–50,000+ depending on declared value |
| Port handling — Jeddah THC | USD 150–250 per unit (est.) = USD 1,200–2,000 total |
| Chokepoint exposure | HIGH — Bab el-Mandeb; Houthi embargo active |

2.2  Scenario B — Melbourne → Khorfakkan (UAE) → Overland to Dammam → Saudi Arabia

PREFERRED ROUTING (on risk basis) No Red Sea exposure. No Hormuz exposure. SASO certification valid at any Saudi port of entry. Key constraint: ~4-week minimum wait at Khorfakkan due to diverted vessel congestion.

Route: Melbourne → Singapore or Port Klang (transhipment) → Indian Ocean → Khorfakkan (Sharjah, UAE) → road approximately 700–800 km → Dammam (King Abdulaziz Port, Eastern Province, Saudi Arabia).

Khorfakkan port is located on the Gulf of Oman coast — accessible from the Indian Ocean without transiting the Strait of Hormuz. It is a 6-berth deep-water facility operated by Gulftainer, with confirmed RoRo capability for H&H cargo.

Dammam (King Abdulaziz Port) has dedicated H&H RoRo berths (27, 28, 29 and 35, 36 with RoRo ramps). Full heavy vehicle discharge capability confirmed.

The Sharjah-Sajaa-Dammam bonded dry port corridor (launched 2026) provides a structured freight pathway for overland transit.

SASO/ZATCA at Dammam: SASO type approval is nationally administered — the owner's certificate is valid at Dammam (or any Saudi port of entry), not Jeddah-specific. ZATCA customs processing applies identically at Dammam.

| Parameter | Detail |
| --- | --- |
| Sea transit to Khorfakkan | 32–40 days Melbourne to Khorfakkan |
| Khorfakkan port wait (current) | ~4 weeks minimum (congestion from Hormuz diversion surge) |
| Overland leg | Khorfakkan → Dammam ~700–800 km; estimated 2–4 days road transit including Saudi border customs |
| Total elapsed time | 65–90 days Melbourne to Dammam |
| Self-drive option | Prime movers are self-propelled — can drive under own power; eliminates lowboy cost. Requires UAE and Saudi oversize convoy permits (3 m wide × 18 m long) |
| Overland cost (est.) | Self-drive: driver costs + permits ~USD 3,000–8,000 total for 8 units |
| Chokepoint exposure | NONE |
| Delivery point note | Delivers to Dammam. If final buyer is in Jeddah, domestic Saudi leg (~1,200 km Dammam → Jeddah) is required — typically buyer's responsibility depending on Incoterms. |

2.3  Scenario C — Melbourne → Khorfakkan (UAE) → Overland to Jeddah

Route: Melbourne → Singapore or Port Klang (transhipment) → Indian Ocean → Khorfakkan → road approximately 2,000–2,200 km → Jeddah.

This scenario uses the same sea leg as Scenario B but extends the overland leg directly to Jeddah, bypassing Dammam. This is relevant because the buyer is confirmed in Jeddah — delivering direct to Jeddah avoids the need for the buyer to arrange a domestic Saudi haulage leg from Dammam.

The prime movers are self-propelled and can drive under their own power on this route. Oversize convoy permits for 3 m wide × 18 m long vehicles are required in both UAE and Saudi Arabia.

| Parameter | Detail |
| --- | --- |
| Sea transit to Khorfakkan | 32–40 days (same as Scenario B) |
| Khorfakkan port wait (current) | ~4 weeks minimum (same constraint as Scenario B) |
| Overland leg | Khorfakkan → Jeddah ~2,000–2,200 km via UAE interior → Saudi border (Ghuwaifat/Al Batha) → Riyadh → Jeddah. Estimated 4–6 days road transit including customs. |
| Total elapsed time | 70–95 days Melbourne to Jeddah |
| Self-drive option | Yes — prime movers self-propelled. Substantially reduces overland cost. |
| Overland cost (est.) | Self-drive: driver costs + permits ~USD 6,000–15,000 total for 8 units (longer leg) |
| Chokepoint exposure | NONE |
| Delivery point | Jeddah — direct delivery to buyer's location without additional Saudi domestic leg. |

PART 3 — SCENARIO COMPARISON

| Element | Scenario A Jeddah via Red Sea | Scenario B Khorfakkan → Dammam | Scenario C Khorfakkan → Jeddah (road) |
| --- | --- | --- | --- |
| Discharge port | Jeddah Islamic Port | Dammam / King Abdulaziz Port | Khorfakkan (UAE), then road |
| Chokepoint exposure | Bab el-Mandeb ACTIVE EMBARGO | NONE | NONE |
| Sea transit | 35–60 days | 32–40 days to Khorfakkan | 32–40 days to Khorfakkan |
| Port wait | N/A | ~4 weeks (Khorfakkan) | ~4 weeks (Khorfakkan) |
| Overland | None | ~700–800 km; 2–4 days | ~2,000–2,200 km; 4–6 days |
| Total elapsed | 45–70 days | 65–90 days | 70–95 days |
| Ocean freight base (144 LM) | USD 29k–65k | USD 29k–65k | USD 29k–65k |
| War risk surcharge | USD 8k–50k+ (active) | NONE | NONE |
| Overland cost (est.) | Nil | USD 3k–8k (self-drive) | USD 6k–15k (self-drive) |
| Delivers to Jeddah directly | Yes | No — Dammam; buyer arranges domestic leg | Yes |
| SASO/ZATCA | Confirmed | Equally valid at Dammam | Border crossing clearance |
| Verdict | ⚠ NOT RECOMMENDED | PREFERRED — lowest risk | VIABLE — direct Jeddah delivery |

MEO recommendation: Progress Scenario C for carrier RFQ — it delivers directly to Jeddah (confirmed buyer location) with no chokepoint exposure. Scenario B remains the fallback if the overland distance for Scenario C presents a permit or logistics obstacle. Do not progress Scenario A until the Houthi maritime embargo on Saudi vessels is lifted.

PART 4 — CARRIERS AND PORT OF MELBOURNE

4.1  Confirmed Carriers — Melbourne to Middle East H&H RoRo

| Carrier | Melbourne Presence | Middle East Ports | Status |
| --- | --- | --- | --- |
| NYK RoRo | Confirmed — Webb Dock West | Jeddah, Aqaba, Jebel Ali, Dammam, Bahrain, Kuwait | CONFIRMED — Australia to Middle East on schedule (updated 19 Nov 2025) |
| WWL (Wallenius Wilhelmsen) | Confirmed — Webb Dock West | Jeddah, Dammam, Jebel Ali, Aqaba, Kuwait | CONFIRMED — all vessels via Cape of Good Hope as at Jul 2026 |
| Höegh Autoliners | Confirmed — multiple AU ports, up to 4×/month | Jebel Ali (direct); Gulf via transhipment | CONFIRMED — Jebel Ali–Singapore direct confirmed; current Red Sea position post-embargo unconfirmed |
| K-Line Global RoRo | Confirmed — Asia-Australia service | Jeddah, Jebel Ali | PROBABLE — Middle East capability confirmed; direct Melbourne export rotation unconfirmed |
| MOL | Australia service confirmed | Jebel Ali, Middle East | PROBABLE — specific Melbourne rotation unconfirmed in this research |
| Grimaldi | Insufficient evidence | N/A | NOT RELEVANT — Grimaldi focuses on Europe/Africa/Americas; remove from consideration |

RFQ should be issued simultaneously to NYK, WWL, and Höegh. Specify: 8 × H&H RoRo units; 18 m × 3 m × 2.8 m; 20,000 kg GVM; degassed ex-fuel tanker (Gas-Free Certificates held); Melbourne export; discharge preference Khorfakkan or Fujairah (primary); confirm H&H deck width allocation for 3 m cargo; request confirmation of whether width billing factor applies.

4.2  Port of Melbourne — AAT Webb Dock West

| Item | Detail |
| --- | --- |
| Terminal | AAT Webb Dock West (formerly MIRRAT) |
| Operator | Australian Amalgamated Terminals Pty Ltd (subsidiary of Qube Holdings). Sale from WWL completed 1 May 2025. |
| Address | 46 Kooringa Way, Port Melbourne VIC 3207 |
| Admin phone | +61 3 8698 6960 |
| Rec & Del Office | +61 3 8698 6970 |
| Email | WD.Dispatch@aaterminals.com.au |
| Berths | 3 |
| Key equipment | 120-tonne gantry crane; 35 hectare footprint; 8,000 m² undercover storage; 2 quarantine wash bays |
| Cargo booking | Placed directly with carrier (NYK, WWL, Höegh) — not with terminal. Terminal handles vessel berthing. |
| NYK contact | australia.roro@nykgroup.com |

PART 5 — INDICATIVE FREIGHT RATES

Rate Data Notice All figures below are indicative — Tier 4/5 sources. H&H RoRo rates on this lane are 100% quote-based. No confirmed carrier tariff is publicly available for Melbourne → Middle East H&H RoRo. A live formal RFQ to NYK, WWL, and Höegh is required before any rate is used in a commercial decision.

| Cost Component | Indicative Range | Basis / Source |
| --- | --- | --- |
| Ocean freight — base rate | USD 200–450 per lane metre | General H&H RoRo market intelligence; Tier 4/5 |
| Ocean freight total (144 LM — length basis) | USD 28,800–64,800 | Indicative; requires live quote |
| Ocean freight total (216 LM — if 3 m width factor applies) | USD 43,200–97,200 | Indicative; carrier-specific |
| BAF (Bunker Adjustment Factor) | +15–25% of base ocean rate | Currently elevated; VLSFO USD 461.54/tonne as at Jan 2026 (Maersk Platts index) |
| CAF (Currency Adjustment Factor) | +1–5% of base ocean rate | AUD/USD volatility; applies if freight in non-USD denomination |
| War risk surcharge — Scenario A only | 0.75–3% of cargo declared value | Insurance Journal 23 Jul 2026; active Houthi embargo |
| War risk cost impact — Scenario A | USD 8,000–50,000+ | Highly variable; depends on declared value |
| Terminal Handling Charge — Melbourne (origin) | ~AUD 200–400 per unit est. | AAT Webb Dock West; H&H premium over standard auto; Tier 5 estimate |
| Terminal Handling Charge — Jeddah (destination) | USD 150–250 per unit est. = USD 1,200–2,000 total | Saudi RoRo port THC; Tier 5 estimate |
| Overland — Scenario B (Khorfakkan → Dammam) | USD 3,000–8,000 total (self-drive) | Convoy permits, driver costs; Tier 5 estimate |
| Overland — Scenario C (Khorfakkan → Jeddah) | USD 6,000–15,000 total (self-drive) | Longer 2,200 km leg; Tier 5 estimate |
| TOTAL ALL-IN ESTIMATE — Scenarios B/C (excl. war risk) | USD 40,000–100,000 | Length-only LM; inc. BAF, port charges, overland; Tier 4/5 |
| TOTAL ALL-IN ESTIMATE — Scenario A | USD 48,000–150,000+ | Includes war risk surcharge at 3% ceiling; highly variable |

5.1  Lane Metre Width Factor — Critical Clarification

Standard lane metre definition: 1 LM = 1 m length × 2 m width. These units are 3 m wide.

On H&H decks on modern PCTCs (WWL Mark IV/V, Höegh Aurora class, NYK LCTC class), 3 m width commonly fits within a single H&H lane allocation — NO width surcharge on dedicated H&H decks.

On older vessels with mixed H&H/car-deck configurations, a 1.5× width factor may apply: 8 × 27 LM = 216 LM vs. 8 × 18 LM = 144 LM — a 50% rate increase.

All RFQs must specify 3 m width explicitly and request written confirmation of H&H deck allocation and whether any width billing factor applies.

PART 6 — DG PRE-BOOKING GATE (REMY)

6.1  Gate Status: Conditionally Cleared

Prepared by: Remy, Compliance Specialist | 27 July 2026 | v2

DG Gate: CONDITIONALLY CLEARED UN 1202, Diesel Fuel, Class 3 Flammable Liquid, Packing Group III — confirmed. Gas-Free Certificates held. Tanks sealed. Framework sound. Booking may proceed to preparation. Carrier booking must not be lodged until certificate validity vs. vessel ETD is confirmed.

6.2  Key DG Findings

Marine pollutant designation — MUST be declared:

UN 1202 Diesel fuel carries a marine pollutant designation under the IMDG Code. Even for a degassed tank, the shipper's declaration and carrier DG notification must reference this. The marine pollutant mark is not required on the vehicles given gas-free status, but the declaration must acknowledge it. This is commonly missed on formerly-DG vehicle shipments.

Certificate validity timing — HIGHEST RISK ADMINISTRATIVE ITEM:

Gas-Free Certificates are valid for 24–48 hours from time of issue (confirm exact validity from each certificate), or until the tank seal is broken — whichever comes first. Tanks are currently sealed. The following will void the seal and require immediate re-testing: opening of any tank hatch, manway, or discharge valve; physical damage breaching tank integrity; any introduction of substance into the tank.

Correct sequencing: (1) Confirm vessel ETD and cargo receival cut-off at Port of Melbourne. (2) Arrange Gas-Free Certificate issue within 24–48 hours before units are presented at the port gate. (3) Do not break tank seals before loading.

6.3  DG Gate — Outstanding Items

| # | Item | Status | Action Required | Owner |
| --- | --- | --- | --- | --- |
| 1 | Last product — UN 1202 confirmed | CLEARED | None | — |
| 2 | Degassing method assessed | CLEARED | Include wash-and-mop method in carrier DG notification | Soren / freight forwarder |
| 3 | Gas-Free Certificate copies obtained by MEO | OUTSTANDING | MEO to obtain copies; confirm issuer, date, LEL readings, validity window | MEO → Owner |
| 4 | Certificate validity vs. vessel ETD | OUTSTANDING | Soren to confirm ETD; verify certificates valid at port gate or schedule re-testing | Soren |
| 5 | Tank seals — no-break instruction issued | CLEARED | Logistics handler to be briefed — no seal breaks before loading | Soren / handler |
| 6 | Marine pollutant flag in declaration | OUTSTANDING | Freight forwarder to include UN 1202 marine pollutant designation in shipper's declaration | Freight forwarder |
| 7 | Carrier DG notification package lodged | OUTSTANDING | Soren to lodge with carrier DG officer once item 3 confirmed | Soren |
| 8 | DAFF biosecurity | LOW RISK — NEW BUILDS | Standard export lodgement by freight forwarder. Brand new vehicles — DAFF inspection unlikely. | Freight forwarder |

PART 7 — SAUDI ARABIA IMPORT REQUIREMENTS

7.1  HS Codes

| Component | HS Code | Description | Indicative GCC Duty |
| --- | --- | --- | --- |
| Volvo prime mover (road tractor for semi-trailer) | 8701.20 | Road tractors for semi-trailers | 5% (indicative — verify on ZATCA FASAH) |
| Fuel tanker semi-trailer | 8716.39 or 8716.40 | Semi-trailers / tank semi-trailers for transport of goods | 5–12% depending on sub-classification — verify on ZATCA |
| If declared as combined unit | 8701 + 8716 | Customs will typically assess as two separate commodity lines | Duty per component |

Note: Saudi Arabia updated SABER Customs Tariff Codes effective 1 January 2026. Updated codes must be verified in the SABER platform before submission.

7.2  SASO / SABER — Per-Consignment Certificate Required

SASO type approval confirmed — but a per-consignment SCoC is also required The owner holds SASO type approval (Product Certificate of Conformity — PCoC). This is confirmed and is not a gate item. HOWEVER: A Shipment Certificate of Conformity (SCoC) must also be registered in SABER for each consignment before the vessel departs Melbourne. Letters of Undertaking are no longer accepted as substitutes (abolished January 2025). Certificates obtained after arrival = regulatory non-compliance and risk re-export. Action: Confirm SCoC is obtained and registered in SABER for this specific shipment before booking is finalised.

7.3  5-Year Rule — HARD GATE

⚠ CRITICAL — Confirm year of manufacture before booking Saudi Arabia applies a 5-year age restriction to heavy transport vehicles exceeding 3.5 tonnes GVW (including semi-trailers). Age is calculated from 1 January of the year of manufacture. For 2026 imports: vehicles must have been manufactured in 2021 or later. These units at 20,000 kg GVM are well inside the rule threshold. If any unit was built before 2021, it CANNOT be imported under standard Saudi customs rules. Owner action required: Confirm year of manufacture for all 8 prime movers and all 8 tanker trailers before advancing bookings.

PART 8 — ACTIONS REQUIRED BEFORE BOOKING

| Priority | Action | Owner | Gate Impact |
| --- | --- | --- | --- |
| 1 — HARD GATE | Confirm year of manufacture for all 8 prime movers and 8 tanker trailers | Owner → MEO | 5-year rule — blocks Saudi import if pre-2021 |
| 1 — HARD GATE | Confirm SABER SCoC (Shipment Certificate of Conformity) is obtained and registered for this consignment before vessel departure | Owner / freight forwarder | Required for Saudi customs clearance |
| 2 — DG GATE | Provide Gas-Free Certificate copies to MEO — issuer, date, validity period, % LEL readings per unit | Owner → MEO | Required for carrier booking package |
| 3 — BOOKING | Issue simultaneous RFQ to NYK, WWL, and Höegh — 8 × 18 m × 3 m × 2.8 m; 20,000 kg; Khorfakkan/Fujairah discharge; H&H deck width confirmation; degassed ex-fuel tanker | Soren (once gates 1–2 resolved) | Drives routing and cost decision |
| 4 — BOOKING | Confirm Khorfakkan 4-week wait with carrier and terminal; explore Fujairah as alternate UAE east coast discharge | Soren | Critical path for total transit time |
| 5 — BOOKING | Obtain UAE and Saudi oversize convoy permits for 3 m × 18 m × 2.8 m vehicle convoy (Scenario C or B overland) | Soren / freight forwarder | Required for self-drive overland leg |
| 6 — DOCUMENTATION | Freight forwarder to include UN 1202 marine pollutant designation in shipper's DG declaration | Freight forwarder | IMDG compliance — commonly missed |
| 7 — DOCUMENTATION | Coordinate Gas-Free Certificate issue date with vessel ETD so certificates are valid at port gate (24–48 hour window) | Soren + freight forwarder | Certificates expire — timing is critical path |

MEO recommends progressing items 1 and 2 immediately in parallel. Items 3–7 follow once year of manufacture and SCoC are confirmed.

The Works  |  Freight Advisory Report  |  27 July 2026  |  Prepared by Soren (Global Freight Specialist) and Remy (Compliance Specialist)  |  Authorised by MEO  |  All indicative rates Tier 4/5 — live RFQ required before commercial commitment