# P&ID Review & LPG Spray Fill Differential Pressure Assessment

Drawing reviewed: 29629-10-01, Rev 1, 28/10/2025

Build: Supagas LPG Rigid Tanker Rehorse

Prepared by: Petra — Process & Instrumentation Engineer, The Works

Date: 19 May 2026

Sources: Drawing 29629-10-01 Rev 1; ADG Code 7th Ed. Class 2.1; AS 1596:2014; Victorian DG Act 1998; ISA 5.1; ISO 5167

Owner-provided context: 300/500/900 kPa pressure behaviours observed; target 700 kPa not achievable; bypass poppet replaced; isolation valve on L3-50 physically present but not shown on approved drawing. — owner-provided, 19 May 2026

## Part 1 — P&ID Drawing Review

### 1.1 Drawing Status

Drawing 29629-10-01 Rev 1 carries Victorian WorkCover Authority approval, stamped "APPROVED — Dangerous Goods." This is registered dangerous goods plant. Any modification — physical or procedural — must be assessed against the Victorian Dangerous Goods Act 1998 and the Equipment (Public Safety) Act (Vic) before implementation.

### 1.2 Drawing Completeness — Issues Found

| # | Issue | Severity | Action Required |
| --- | --- | --- | --- |
| 1 | Isolation valve on L3-50 pipework not shown on approved drawing | HIGH | As-built revision required. Submit Rev 2 to WorkCover for noting or re-approval. |
| 2 | Line tag "L3-50" not clearly labelled — cross-reference against nodes table required | Medium | Confirm line tag against physical installation. Update drawing if incorrect. |
| 3 | Torque settings table not populated on Rev 1 (marked "AS-BUILT ONLY") | Low | Populate on as-built revision before final approval. |

The missing isolation valve on L3-50 is a compliance issue. The approved drawing does not reflect the as-installed condition. This must be corrected before any operating procedure changes are implemented.

### 1.3 Process Paths Identified

| Line | Service | Size | Notes |
| --- | --- | --- | --- |
| Spray Fill (A–B) | Liquid LPG spray fill | Ø50 | Main spray fill delivery |
| Vapour (C–D) | Vapour equalisation | — | Return to source vessel |
| Pump Suction (E–F) | Liquid suction from tanker | Ø40 | Via strainer/air eliminator |
| Pump Discharge (P–J) | Pressurised liquid discharge | Ø50 | Via turbine meter, differential valve |
| Vapour Bypass (G–K) | Bypass recirculation | — | Via Ebsray RV19 VRS bypass valve |
| Return/Relief (P–Q) | Return / recirculation | Ø50 | L3-50 line |

The Corken Z3200 is a positive displacement (PD) hydraulic motor-driven pump. PD pumps deliver a fixed flow rate at any given speed; pressure is determined entirely by system resistance. This is the key to understanding the three observed pressure states.

## Part 2 — Differential Pressure Analysis

### 2.1 The Three Pressure States

### State 1 — 300 kPa (Bypass wound out / active)

The Ebsray RV19 VRS is a spring-loaded differential pressure relief valve. "Wound out" reduces spring pre-load — the valve opens at lower differential pressure. At 300 kPa, the bypass opens and recirculates pump output back to suction. The pump cannot build beyond this threshold. This is a bypass valve setting issue, not a pump or system fault.

Confirmed: hydraulically correct.

### State 2 — 500 kPa (Bypass closed, L3-50 open)

With the bypass closed, L3-50 provides a parallel return path — a low-resistance route from the high-pressure discharge side back to suction. The PD pump delivers flow into the path of least resistance. Flow divides between the spray nozzle and the L3-50 return. The system self-balances at 500 kPa. L3-50 is acting as a hydraulic bypass across the pump.

Confirmed: hydraulically correct.

### State 3 — 900 kPa (Bypass closed, L3-50 closed)

All recirculation paths eliminated. All pump output must pass through the spray nozzle. Resistance is maximum; differential pressure rises to 900 kPa.

Confirmed: hydraulically correct. This result is diagnostic.

### 2.2 What the 900 kPa Result Proves

The 900 kPa result eliminates the following as root causes:

Pump wear or underperformance — a worn pump cannot reach 900 kPa

Bypass valve body wear or internal leakage — a leaking bypass body would cap State 3 below 900 kPa

Spray nozzle blockage

Major downstream restriction fault

The bypass poppet wear question is resolved. New poppet snug + State 3 at 900 kPa = bypass valve body wear is not the cause. The L3-50 recirculation loop is the sole cause of the 500 kPa ceiling.

## Part 3 — Solution Assessment

### 3.1 Interim Solution — Throttling L3-50

The proposed interim approach is hydraulically valid. Gradually closing L3-50 will restrict return flow, increase system resistance, and raise differential pressure toward 700 kPa.

LPG-specific concern — throttling service on L3-50: L3-50 is likely a ball valve (DN50 FNPT). Ball valves are quarter-turn on/off devices — not designed for throttling service. In LPG service, partial closure creates a high-velocity restriction where LPG can flash to vapour, causing rapid ball/seat erosion, control instability, and the valve becoming stuck.

The L3-50 isolation valve must not be used as a permanent throttling device. Single-use partial closure to find the 700 kPa setpoint and mark the position is acceptable. Continuous throttled operation is not.

### 3.2 Permanent Solutions — Ranked

| Option | Assessment | Recommendation |
| --- | --- | --- |
| Restriction orifice (fixed) | Best permanent solution for LPG. Passive, no moving parts, no maintenance, no operator error. | RECOMMENDED — first choice |
| Control valve (modulating) | Active control, flexible. Justified only if differential pressure must vary dynamically. | Second choice if variable setpoint needed |
| Lock ball valve in position | Viable interim only. Ball valve wear in throttled LPG service is a known failure mode. | Interim only — not permanent |

### 3.3 Restriction Orifice Sizing

The "20–25 mm" estimate must be calculated, not assumed. Correct sizing requires:

Confirmed pump flow rate through L3-50 at operating condition (from Corken Z3200 pump curve)

LPG density and vapour pressure at operating temperature

Required pressure drop across the orifice to achieve 700 kPa system differential

Calculation to ISO 5167 or equivalent

Petra will size the orifice once flow rate, operating temperature, and flow split data are provided.

## Part 4 — Compliance Assessment

### 4.1 WorkCover Implications

| Action | WorkCover Obligation |
| --- | --- |
| Operating L3-50 as throttle (procedural change) | Assess as modification under DG Act. Likely requires MOC process and WorkCover notification. Do not implement permanently without this assessment. |
| Installing restriction orifice in L3-50 line | Physical modification to registered DG plant. Requires design registration revision and WorkCover approval BEFORE installation. |
| As-built discrepancy — missing isolation valve | Must be corrected regardless. Issue drawing Rev 2. Submit to WorkCover for noting. |

### 4.2 Recommended Compliance Sequence

Step 1 — Immediately: Issue drawing revision 29629-10-01 Rev 2 showing the L3-50 isolation valve as installed.

Step 2 — Before interim operation: Conduct MOC assessment for using L3-50 to set differential pressure. Consult WorkCover Victoria or a registered DG adviser.

Step 3 — Before permanent modification: Submit revised P&ID showing restriction orifice to WorkCover for approval before fabrication.

Step 4 — On approval: Fabricate and install orifice to calculated size. Update as-built drawing.

### 4.3 Safe Operating Procedure — Setpoint Establishment (Interim Use Only)

The following procedure is for finding and marking the 700 kPa setpoint only — not for continuous operation:

1. Confirm bypass valve is wound in (closed) and functional

2. Confirm L3-50 isolation valve is fully open (normal start position)

3. Start pump — system will stabilise at approximately 500 kPa

4. Slowly close L3-50 isolation valve — observe differential pressure gauge

5. Stop closing when differential pressure reaches 700 kPa

6. Note valve position only — do not operate continuously in this partially closed position

7. Use position data to specify the restriction orifice

## Part 5 — Summary

### Can the valve (L3-50) be used?

YES — once, to find the 700 kPa setpoint and specify the orifice.
NO — as a permanent throttling device. Ball valves in LPG throttling service will fail.

### Is the analysis correct?

YES. The hydraulic reasoning is correct in every material respect. The analysis does not address WorkCover compliance or ball valve throttling service — these are Petra's additions.

### Outstanding Items

| Item | Owner | Action |
| --- | --- | --- |
| As-built drawing revision (missing isolation valve) | Owner + Liquip Victoria | Issue Rev 2 immediately |
| MOC assessment for interim operating procedure | Owner + DG adviser | Before operating L3-50 throttled |
| Orifice sizing calculation | Petra | Provide pump flow rate + operating temperature |
| Drawing revision for orifice | Petra + Liquip Victoria | After sizing, before fabrication |
| WorkCover re-approval for orifice | Owner | Submit revised drawing before fabrication |

Petra — Process & Instrumentation Engineer, The Works | 19 May 2026

Note: Petra advises. Statutory sign-off on WorkCover submissions requires a licensed CPEng — owner to engage where mandated by the Equipment (Public Safety) Act (Vic).