# PTO & PTO Pump Specification Template

## Fuel Pumping Vehicles — All Makes and Models

Prepared by: Harlow (Heavy Vehicle Body Engineer) + Petra (Process & Instrumentation Engineer), The Works

Date: 19 May 2026

Purpose: Guide young engineers through complete PTO and PTO pump specification for fuel pumping vehicles. Complete every section in order.

HOW TO USE: Work top to bottom. Sections 1–3 gather vehicle facts. Section 4 defines pump system requirements. Section 5 selects the PTO. Section 6 sizes and selects the pump. Section 7 checks installation. Section 8 records the final specification. Never assume or estimate without noting it — flag anything you cannot confirm.

## Section 1 — Vehicle Information

Establish the chassis before anything else. Every PTO selection depends on the transmission.

| Field | Entry | Source / Notes |
| --- | --- | --- |
| Vehicle make |  |  |
| Vehicle model |  |  |
| Model year |  |  |
| VIN / chassis number |  | Confirm from compliance plate |
| Engine make and model |  |  |
| Engine max power | kW @ ___ rpm | From OEM spec sheet |
| Engine max torque | Nm @ ___ rpm | From OEM spec sheet |
| Engine governed speed (max RPM) | rpm | From OEM spec sheet |
| Transmission make and model |  | e.g. Allison 3000, ZF 9-speed, Eaton Fuller |
| Transmission type | ☐ Manual ☐ AMT ☐ Automatic ☐ Torque Converter |  |
| Transmission serial number |  | From transmission ID plate |
| Number of PTO ports available |  | Check OEM Body Builder Instructions |
| PTO port locations available | ☐ Top ☐ Side LH ☐ Side RH ☐ Rear | Check OEM BBI |
| GVM | kg |  |
| BBI version used |  | Record document title and revision |

## Section 2 — Application Information

Define the fuel type and duty cycle — these drive every subsequent decision.

| Field | Entry | Notes |
| --- | --- | --- |
| Application type | ☐ Fuel delivery ☐ Bulk fuel ☐ Aviation ☐ LPG ☐ Other: |  |
| Fuel / product ADG class | ☐ Class 2.1 (LPG) ☐ Class 3 (flammable liquid) ☐ Class 6 ☐ Class 8 |  |
| Fluid being pumped |  | e.g. diesel, petrol, avgas, LPG |
| Fluid vapour pressure at max operating temp | kPa abs | Required for NPSH — see Section 4 |
| Fluid density at operating temp | kg/m³ | Diesel ≈ 820–840 kg/m³ |
| Fluid viscosity at operating temp | cSt | Required for pump selection |
| Max operating temperature | °C |  |
| Min ambient temperature (cold start) | °C | Affects viscosity and priming |
| Required delivery flow rate | L/min | Owner/customer specification |
| Required operating pressure (differential) | kPa | Owner/customer specification |
| Duty cycle | ☐ Continuous (>30 min) ☐ Intermittent ☐ Short burst (<5 min) | Drives PTO and pump thermal rating |
| Expected daily operating hours | hr/day | For service life |

## Section 3 — Existing System (Rehorse / Modification Only)

Complete only if modifying an existing pump system. Skip entirely for new builds.

| Field | Entry | Notes |
| --- | --- | --- |
| Existing PTO make and model |  |  |
| Existing PTO ratio | :1 |  |
| Existing pump make and model |  |  |
| Existing pump displacement | cc/rev |  |
| Known issues with existing system |  |  |
| Reason for replacement / modification |  |  |
| Existing drawings or approvals to carry over |  | Note document numbers |

## Section 4 — System Performance Requirements

Define the numbers the pump must hit. Confirm these before proceeding to Sections 5 and 6.

### 4.1  Flow and Pressure Requirements

| Parameter | Required Value | Confirmed by | Date |
| --- | --- | --- | --- |
| Minimum delivery flow rate | L/min |  |  |
| Maximum delivery flow rate | L/min |  |  |
| Operating differential pressure | kPa |  |  |
| Maximum allowable system pressure | kPa |  |  |
| Relief valve set pressure | kPa |  | Must be ≤ max system pressure |

### 4.2  Pump Power Requirement

PUMP SHAFT POWER (kW) = [Flow (L/min) × Pressure (kPa)] ÷ [60,000 × Pump efficiency]

Typical pump efficiency:

Gear pump:    0.80 – 0.88

Vane pump:    0.80 – 0.88

Piston pump:  0.85 – 0.93

Example:  Flow = 200 L/min,  Pressure = 700 kPa,  Efficiency = 0.85

Power = (200 × 700) ÷ (60,000 × 0.85) = 2.75 kW

| Calculation Step | Value | Units |
| --- | --- | --- |
| Required flow |  | L/min |
| Operating pressure |  | kPa |
| Assumed pump efficiency |  | (decimal) |
| Calculated pump shaft power |  | kW |
| Add 25% service factor (× 1.25) |  |  |
| MINIMUM PTO OUTPUT POWER REQUIRED |  | kW |

### 4.3  NPSH Check  (Suction Capability — Critical for Volatile Fuels)

NPSHA must exceed the pump manufacturer's NPSHR by at least 0.5 m. If it does not: reposition the pump lower, enlarge the suction line, or use a flooded suction arrangement.

NPSHA = (Atmospheric pressure − Fluid vapour pressure) ÷ (ρ × g)  +  Static head  −  Suction losses

ρ = fluid density (kg/m³)    g = 9.81 m/s²

Static head is POSITIVE if pump is below the fluid surface (flooded suction)

Static head is NEGATIVE if pump must lift the fluid (suction lift)

| Parameter | Value | Units |
| --- | --- | --- |
| Atmospheric pressure | 101.3 | kPa abs (standard) |
| Fluid vapour pressure at max operating temp |  | kPa abs |
| Fluid density |  | kg/m³ |
| Static suction head (+ flooded / − lift) |  | m |
| Estimated suction line losses |  | m |
| CALCULATED NPSHA |  | m |
| Pump manufacturer's NPSHR |  | m |
| Margin (NPSHA − NPSHR) — must be ≥ 0.5 m |  | m |
| NPSH check result | ☐ PASSED ☐ FAILED — redesign suction arrangement |  |

## Section 5 — PTO Selection

The OEM Body Builder Instructions for the specific transmission MUST be obtained before selecting a PTO. Installing an unapproved PTO voids the transmission warranty and can cause transmission failure.

### 5.1  Transmission PTO Requirements

| Field | Entry | Source |
| --- | --- | --- |
| OEM approved PTO list for this transmission | List approved part numbers: | From BBI |
| PTO mounting position selected | ☐ Top ☐ Side LH ☐ Side RH ☐ Rear | From BBI |
| PTO engagement method | ☐ Mechanical ☐ Pneumatic ☐ Electric ☐ Hydraulic | From BBI / application |
| PTO engagement — vehicle must be | ☐ Stationary only ☐ Moving permitted | From BBI |
| Transmission max PTO torque at this port | Nm | From BBI — do not exceed |
| PTO output rotation direction | ☐ CW ☐ CCW (viewed from PTO output shaft) | Confirm with pump requirement |

AUTOMATIC AND AMT TRANSMISSIONS (Allison, ZF Traxon, Eaton UltraShift): PTO engagement conditions, torque limits, and approved PTO lists vary by software version. Always confirm with the transmission OEM or dealer before specifying.

### 5.2  Required PTO Output Speed

PTO OUTPUT SPEED (rpm)  =  Engine RPM  ×  PTO Ratio

Common ratios: 0.55:1  0.68:1  0.80:1  1.00:1  1.05:1  1.25:1

Target: PTO output speed must fall within the pump's rated speed range (see Section 6).

| Parameter | Value | Units |
| --- | --- | --- |
| Target pump operating speed |  | rpm |
| Engine RPM at target pump speed |  | rpm |
| Required PTO ratio (pump rpm ÷ engine rpm) |  | :1 (calculated) |
| Selected PTO ratio (nearest available) |  | :1 (from catalogue) |
| Actual PTO output speed at operating engine RPM |  | rpm |
| Is actual speed within pump rated range? | ☐ Yes ☐ No — select different ratio |  |
| PTO output speed at engine idle |  | rpm — check pump min speed |
| PTO output speed at engine max governed RPM |  | rpm — must not exceed pump max |

### 5.3  PTO Torque Check

PTO OUTPUT TORQUE (Nm)  =  Pump shaft power (kW)  ×  9549  ÷  PTO output speed (rpm)

Apply 1.25 service factor:    Torque required × 1.25  ≤  Transmission PTO torque limit

| Parameter | Value | Units |
| --- | --- | --- |
| Pump shaft power (from Section 4.2) |  | kW |
| PTO output speed |  | rpm |
| Calculated PTO output torque |  | Nm |
| With 1.25 service factor |  | Nm |
| Transmission PTO torque limit (from BBI) |  | Nm |
| Margin (limit − required with SF) |  | Nm — must be positive |
| Torque check result | ☐ PASSED ☐ FAILED — review power requirements or PTO |  |

### 5.4  PTO Selection Record

| Field | Entry |
| --- | --- |
| Selected PTO make |  |
| Selected PTO model / part number |  |
| PTO ratio | :1 |
| PTO output torque rating | Nm |
| PTO output speed at operating RPM | rpm |
| PTO mounting position |  |
| Engagement method |  |
| OEM BBI approval confirmed | ☐ Yes — BBI reference: |
| Transmission warranty implications checked | ☐ Yes ☐ No — investigate before ordering |

## Section 6 — PTO Pump Selection

### 6.1  Pump Type Selection Guide

| Pump Type | Best For | Not Suitable For | Typical Max Pressure |
| --- | --- | --- | --- |
| External gear pump | Clean fuels (diesel, petrol); robust; simple; low cost | High-viscosity cold fluids; fluids with solids | 200–250 bar |
| Internal gear / gerotor | Low-noise; moderate pressure; self-priming | Abrasive fluids | 100–200 bar |
| Vane pump | Smooth flow; low pulsation; moderate pressure | Abrasive or high-viscosity fluids | 100–175 bar |
| Axial piston pump | High pressure; variable displacement; efficient | High cost; not for low-viscosity fuels at high speed | 350–450 bar |
| Liquid ring / regenerative | LPG; volatile liquids; vapour-tolerant | High pressure applications | 15–20 bar |
| Centrifugal | High flow, low pressure | Any application requiring self-prime; high ΔP | 5–10 bar |

Pump type selected: ____________________________   Reason: ______________________________________________

### 6.2  Pump Displacement Sizing

PUMP DISPLACEMENT (cc/rev) = [Flow (L/min) × 1000] ÷ [PTO speed (rpm) × Volumetric efficiency]

Typical volumetric efficiency:

Gear pump:    0.88 – 0.94

Vane pump:    0.88 – 0.92

Piston pump:  0.92 – 0.97

| Calculation Step | Value | Units |
| --- | --- | --- |
| Required flow at operating condition |  | L/min |
| PTO output speed (from Section 5) |  | rpm |
| Assumed volumetric efficiency |  | (decimal) |
| CALCULATED DISPLACEMENT |  | cc/rev |
| Selected pump displacement (nearest catalogue size) |  | cc/rev |
| Actual flow at selected displacement and speed |  | L/min — verify ≥ required |
| Pump max rated speed |  | rpm — verify PTO speed ≤ this |
| Pump min rated speed |  | rpm — verify PTO idle speed ≥ this |
| Pump max rated pressure |  | kPa — verify ≥ system max pressure |

### 6.3  Rotation and Drive Coupling

| Field | Entry | Notes |
| --- | --- | --- |
| Required pump rotation direction | ☐ CW ☐ CCW (from drive end) | Must match PTO output rotation |
| PTO output rotation confirmed | ☐ CW ☐ CCW | From Section 5 |
| Rotation match confirmed | ☐ Yes ☐ No — specify adaptor or alternate pump |  |
| Pump inlet port size and type |  | e.g. 2" BSP female |
| Pump outlet port size and type |  | e.g. 1-1/4" BSP female |
| Drive shaft / flange type | ☐ SAE A 2-bolt ☐ SAE B 2-bolt ☐ SAE C 2-bolt ☐ SAE B 4-bolt ☐ Other | Must match PTO output flange |
| Coupling / adaptor required | ☐ Yes — specify: ☐ No |  |

### 6.4  Fluid Compatibility Check

| Check | Result | Notes |
| --- | --- | --- |
| Pump seals compatible with fuel/product | ☐ Yes ☐ No — specify alternate seal material | e.g. Viton seals for aromatic fuels |
| Pump body material compatible with product | ☐ Yes ☐ No | Check for ethanol blends; LPG compatibility |
| Pump suitable for ADG class of product | ☐ Yes ☐ No | Confirm with pump manufacturer |
| ATEX / hazardous area rating required | ☐ Yes — zone: ___ ☐ No | Check AS/NZS 60079 zone classification |
| Trade measurement (NMI) approval required | ☐ Yes — note requirements ☐ No | Required if used for sale of fuel |

### 6.5  Pump Selection Record

| Field | Entry |
| --- | --- |
| Selected pump make |  |
| Selected pump model / part number |  |
| Pump displacement | cc/rev |
| Pump max speed | rpm |
| Pump max pressure | kPa |
| Actual flow at operating speed | L/min |
| Seal material |  |
| Drive configuration |  |
| Fluid compatibility confirmed | ☐ Yes |
| Manufacturer data sheet attached | ☐ Yes ☐ No |

## Section 7 — Installation Checks

### 7.1  Clearance and Fitment

| Check | Result | Notes |
| --- | --- | --- |
| PTO clears chassis cross members | ☐ Yes ☐ No — modify |  |
| PTO clears exhaust system | ☐ Yes ☐ No — heat shield / reroute |  |
| PTO clears driveline and driveshafts | ☐ Yes ☐ No |  |
| PTO clears cab tilt mechanism (forward cab) | ☐ Yes ☐ No | CRITICAL — check full tilt arc |
| Pump mounting location identified |  |  |
| Pump mounting bracket required | ☐ Yes — design required ☐ No |  |
| Pump accessible for maintenance | ☐ Yes ☐ No — revise location |  |
| Driveshaft / coupling alignment within tolerance | ☐ Yes ☐ No — realign | Per coupling manufacturer spec |

### 7.2  Hydraulic Lines

| Check | Result | Notes |
| --- | --- | --- |
| Suction line diameter adequate for flow rate | ☐ Yes ☐ No | Max suction velocity: 1.0–1.5 m/s |
| Suction line as short and direct as possible | ☐ Yes ☐ No — revise routing | Minimise bends; no high points |
| Suction line rated for vacuum (not just pressure) | ☐ Yes ☐ No | Must not collapse under suction |
| All hoses rated for product and pressure | ☐ Yes ☐ No | Check against ADG class |
| Hose routing avoids heat sources | ☐ Yes ☐ No — reroute / shield |  |
| Hose routing avoids sharp edges and chafe points | ☐ Yes ☐ No — add protection |  |
| Bonding and earthing to chassis | ☐ Yes ☐ No — install bonding strap | ADG Code requirement for Class 3 / LPG |

### 7.3  Controls

| Check | Result | Notes |
| --- | --- | --- |
| PTO engagement control type | ☐ Dash-mounted switch ☐ Remote ☐ PTO control module |  |
| PTO engagement interlock required | ☐ Yes — specify: ☐ No | e.g. park brake must be applied |
| Overspeed protection provided | ☐ Yes ☐ No | Protect pump from engine overspeed |
| Over-pressure relief valve fitted | ☐ Yes ☐ No — install | Set pressure = Section 4.1 |
| Deadman / emergency stop | ☐ Yes ☐ No | ADG Code requirement for Class 3 / LPG |

## Section 8 — Final Specification Summary

Complete this section last. This is the formal record of what was specified and approved.

| Item | Specification |
| --- | --- |
| Vehicle | Make: _______ Model: _______ Year: _______ VIN: _______ |
| Transmission | Make / model: _______________________ Serial: _______________ |
| PTO make / model |  |
| PTO ratio | :1 |
| PTO output speed at operating RPM | rpm |
| PTO torque check | ☐ Passed — margin: _____ Nm |
| PTO OEM BBI approval | ☐ Confirmed — BBI ref: ________________________ |
| Pump make / model |  |
| Pump displacement | cc/rev |
| Pump type |  |
| Actual flow at operating RPM | L/min |
| Operating pressure | kPa |
| NPSH check | ☐ Passed — margin: _____ m |
| Fluid compatibility confirmed | ☐ Confirmed |
| Seal material |  |
| Rotation confirmed — PTO and pump match | ☐ Yes |
| SAE drive flange matched | ☐ Yes |
| All installation clearances | ☐ All passed |
| Relief valve set pressure | kPa |
| Earthing / bonding specified | ☐ Yes |
| Prepared by | Name: ___________________________ Date: ________________ |
| Reviewed by | Name: ___________________________ Date: ________________ |
| Approved by | Name: ___________________________ Date: ________________ |

## Appendix A — Common PTO Ratios by Transmission

| Transmission | Common PTO Ratios Available | Notes |
| --- | --- | --- |
| Allison 3000 Series | 0.55, 0.68, 0.80, 0.87, 1.00 | Multiple ports; confirm ratio with Allison BBI |
| Allison 4000 Series | 0.55, 0.68, 0.80, 1.00, 1.05 | Higher torque capacity than 3000 |
| ZF 9-speed (Traxon / AS Tronic) | 0.55, 0.68, 1.00 | AMT — engagement conditions apply |
| Eaton Fuller (manual) | 0.68, 0.80, 0.87, 1.00, 1.05 | Mechanical engagement; multiple port options |
| Eaton UltraShift (AMT) | 0.68, 1.00 | AMT — check software version for PTO support |
| Isuzu 6-speed AMT | Confirm with Isuzu BBI | Limited third-party PTO availability |
| Volvo I-Shift | Confirm with Volvo BBI | Volvo-specific PTO; OEM approval required |

Always confirm available ratios against the specific transmission serial number — ratios vary by model year and specification.

## Appendix B — Typical Pump Operating Speed Ranges

| Pump Type | Min Speed (rpm) | Max Speed (rpm) | Notes |
| --- | --- | --- | --- |
| External gear pump (<25cc) | 400 | 3000 | Check manufacturer spec for each model |
| External gear pump (>50cc) | 300 | 2500 |  |
| Vane pump | 600 | 2500 | Minimum speed critical for vane loading |
| Axial piston pump | 500 | 3500 |  |
| Corken Z-series (LPG) | 300 | 900 | Hydraulic motor driven — confirm with Corken |
| Blackmer sliding vane | 200 | 600 | Fuel transfer — confirm against specific model |
| Ebsray (fuel / LPG) | 300 | 1750 | Confirm against specific model |

## Appendix C — Key References and Standards

| Document | Scope |
| --- | --- |
| OEM Body Builder Instructions — transmission specific | PTO approval, torque limits, engagement conditions |
| OEM Body Builder Instructions — vehicle chassis | Mounting zones, prohibited areas, hydraulic line routing |
| ADG Code 7th Edition | Dangerous goods transport requirements by class |
| AS 1596:2014 | LPG storage and handling |
| AS 2809.1 / AS 2809.2 | Road tank vehicles for dangerous goods |
| AS/NZS 60079 | Hazardous area classification — pump and motor selection |
| NMI (National Measurement Institute) | Trade measurement approval for metering applications |
| Pump manufacturer data sheet | Displacement, speed range, pressure, seal compatibility |
| Transmission OEM engineering data | PTO torque ratings, approved PTO list, engagement windows |

Harlow — Heavy Vehicle Body Engineer  |  Petra — Process & Instrumentation Engineer  |  The Works  |  19 May 2026

This template is a specification aid. All final designs must be verified against current OEM BBI documentation and applicable Australian Standards. Calculations are for guidance — confirm against manufacturer data before ordering.