Quality System — Baseline Analysis & Data Improvement Plan

Aviation Manufacturing  |  June 2026  |  Internal Working Document

# Purpose

This document establishes a baseline of quality performance using repair action data captured to June 2026, identifies hotspots by cell, defect type, and job, and defines specific improvements to data collection so that future analysis is faster, more accurate, and more actionable.

# Section 1 — Dataset Overview

| Metric | Value |
| --- | --- |
| Total repair actions analysed | 4,964 |
| Date range | September 2024 – June 2026 |
| Primary site | Aviation (4,127) and Offline Aviation (789) |
| Status: Complete | 4,206 (85%) |
| Status: To Do (open) | 166 |
| Status: Pending PCA / post-ship | 53 |
| Status: Won't Do / Hold / Duplicate | 539 |

► 539 "Won't Do / Hold / Duplicate" records should be audited — some may be deferred issues never properly resolved.

# Section 2 — Volume Trend

Repair action volume has grown 72% over 18 months — faster than business growth alone. The process is not keeping pace with scale.

| Month | Actions | Month | Actions |
| --- | --- | --- | --- |
| Oct 2024 | 210 | Jan 2026 | 309 |
| Nov 2024 | 154 | Feb 2026 | 344 |
| Dec 2024 | 128 | Mar 2026 | 362 ← peak |
| Jan 2025 | 215 | Apr 2026 | 276 |
| Feb 2025 | 226 | May 2026 | 249 |
| Mar 2025 | 267 | Jun 2026 | 195 |
| Jul 2025 | 349 ← prior peak |  |  |

► Action: Set a monthly volume target. Suggested: below 150 within 12 months. Without a target, trend data has no meaning.

# Section 3 — Defect Type Analysis

## 3.1 Top Concern Categories

| Concern | Count | % of Total |
| --- | --- | --- |
| Missing | 1,625 | 33% |
| Loose | 177 | 4% |
| Not fitted | 120 | 2% |
| Incomplete | 85 | 2% |
| Not secured | 79 | 2% |
| Not sealed | 56 | 1% |
| Not filed | 56 | 1% |
| Not working | 41 | 1% |
| Leaking | 37 | 1% |
| Wrong / Incorrect | 32 | 1% |
| Blank / Not recorded | 149 | 3% |

► "Missing" at 33% is a BOM completeness and checklist failure — preventable at the desk, not on the floor.

## 3.2 Initiator Breakdown

| Initiator | Count | Meaning |
| --- | --- | --- |
| Q — Quality Inspection | 2,895 | Caught by our inspection process |
| C — Customer | 392 | Reached the customer before detection |
| V — VASS | 90 | Caught at VASS inspection |
| S — Safety | 80 | Safety-related finding |
| M — Manufacturing | 20 | Self-identified on the floor |
| Not recorded | 1,421 | Unknown origin — data gap (29%) |

► 392 customer-initiated issues in aviation refuelling equipment is a significant quality and reputational risk. This number must trend to zero.

## 3.3 Recurring Part Issues — Systemic BOM Failures

| Part | Occurrences | Recommended Action |
| --- | --- | --- |
| Build checklist incomplete | 29 | Enforce checklist as mandatory closure step |
| Spare wheel | 21 | Add to every relevant standard BOM |
| Fuel circuit plate/label | 14 | Add to every relevant standard BOM |
| Ladder | 14 | Add to every relevant standard BOM |
| Filter vessel elements | 11 | Clarify supply/fit responsibility in BOM |
| Weighbridge docket | 10 | Add to closing documentation checklist |
| Flameproofing | 10 | Add to every relevant standard BOM |
| Control panel nutserts | 9 | Add to every relevant standard BOM |
| 5th wheel VASS label | 9 | Add to closing documentation checklist |
| EWP compliance plate | 9 | Engineering to ensure current standard is used |
| Wheel nut indicators | 7 | Add to every relevant standard BOM |
| Pressure vessel door seal | 7 | Add to every relevant standard BOM |
| Soak test form | 7 | Add to closing documentation checklist |

► Implement within 2 weeks. Each is a one-time correction that eliminates a recurring defect permanently.

# Section 4 — Hotspot Analysis

## 4.1 By Cell

| Cell | Repair Actions | Notes |
| --- | --- | --- |
| Manufacturing Aviation — General* | 2,284 | Data attribution gap — must be fixed |
| Manufacturing Aviation — C8-9 | 1,061 | Highest specific cell — investigate |
| Engineering (Mech) | 352 |  |
| Engineering (Documentation) | 289 |  |
| Sales | 262 | Spec/handover related |
| Quality | 244 | Assigned to quality for resolution |
| Manufacturing Aviation — C1-4 | 179 |  |
| Supplier (Stores/Purchasing) | 176 | Supply chain originated |
| Manufacturing Aviation — C5-7 | 174 |  |
| Paint | 168 |  |
| Manufacturing (WO) — Fabrication | 132 |  |
| Engineering (Elect) | 125 |  |
| Manufacturing Aviation — C10 | 120 |  |
| Flame Proofing | 63 |  |

*"General" = defect not assigned to a specific cell. 46% of all records cannot be located to a cell. See Section 5.

## 4.2 Top Jobs by Repair Action Count

| Job Number | Repair Actions |
| --- | --- |
| 801197 | 91 |
| 801153 | 87 |
| 801174 | 85 |
| 801212 | 83 |
| 801261 | 80 |
| 28567 | 76 |
| 801154 | 74 |
| 801125 | 69 |
| 801186 | 67 |
| 801175 | 67 |

► Run a retrospective on the top 5 jobs: product type, cell, dominant defect categories.

## 4.3 Age of Open Items

| Category | Count |
| --- | --- |
| Open items (To Do + Pending post-ship) | 219 |
| Open more than 30 days | 1,062 |
| Open more than 90 days | 298 |
| Open more than 180 days | 108 |
| Maximum days open | 506 days |

► 108 items open more than 180 days require immediate individual review. An item open 506 days is an unresolved structural problem.

## 4.4 Root Cause Closure Rate

| Metric | Value |
| --- | --- |
| Records with root cause owner documented | 108 |
| Records with root cause blank | 4,856 |
| Root cause closure rate | 2% |

► Most significant process gap: fixing problems without asking why. The same defects will recur indefinitely without root cause closure.

## 4.5 Repair Time — Not Being Recorded

Only 3 of 4,964 records have repair time recorded. Total logged: 2 hours. The true cost of poor quality in labour hours is invisible. Without this data there is no business case for investment in prevention.

# Section 5 — Data Collection Improvements

## 5.1 Mandatory Fields

| Field | Current State | Required State | Why |
| --- | --- | --- | --- |
| Initiator | Blank on 29% | Mandatory at creation | Determines where issues are caught in the process |
| Label (Cell) | "General" 46% of time | Mandatory specific cell | Cannot fix what you cannot locate |
| Root Cause Owner | Blank on 98% | Mandatory before closure | Without this, every fix is one-off |
| Repair Time (Hours) | Recorded on 3 records | Mandatory at closure | Makes cost of poor quality visible |
| Part Number | Largely blank | Mandatory where applicable | Enables BOM and supply chain analysis |

## 5.2 Retire the "General" Label

"Manufacturing Aviation — General" must be retired. It absorbs 46% of all repair actions and makes cell-level analysis impossible. Replace with a structured label set mirroring actual cell/area structure. "General" as an escape hatch should not exist.

## 5.3 Standardise the Concern Field — Controlled Dropdown

| Category | Examples |
| --- | --- |
| Missing — not in BOM | Part not specified in BOM |
| Missing — BOM correct, not fitted | Part specified, not installed |
| Incorrect — wrong part | Wrong specification fitted |
| Incorrect — wrong sealant/fastener | Wrong material used |
| Loose / Not secured | Fasteners, fittings, guards |
| Not sealed / Leaking | Seals, gaskets, connections |
| Not working / Functional failure | Electrical, pneumatic, hydraulic |
| Damaged | Transport, handling, manufacturing damage |
| Documentation missing | Manuals, certificates, forms |
| Documentation incorrect | Wrong version, incomplete |
| Not painted / Finish defect | Paint, coating, surface |
| Clash / Interference | Physical conflict between components |
| Compliance — label/plate | Regulatory labels, compliance plates |
| Compliance — certification | Missing or out-of-date certificates |
| Other (with mandatory description) |  |

## 5.4 Root Cause Category Field

| Category | Description |
| --- | --- |
| BOM incomplete | Item not in bill of materials |
| BOM incorrect | Wrong item or quantity in BOM |
| Engineering drawing error | Error in drawing released to floor |
| Engineering package late | Late release caused downstream issue |
| Assembly omission | BOM correct, item not fitted |
| Wrong specification | Built to wrong version/spec |
| Supplier defect | Part received defective or wrong |
| Supplier late delivery | Material not available when needed |
| Process not followed | Correct process exists, not followed |
| Process does not exist | No process defined |
| Documentation not filed | Physical/digital filing failure |
| Sales specification error | Incorrect spec at order stage |
| Customer-requested change | Post-order scope change |
| Transport damage | Damaged after leaving facility |

## 5.5 Add Job Type Field

Aviation Refueller

Aviation Dispenser / Hydrant Dispenser

Fuel Tanker

Bulk Liquid Transport

Pump Skid

Other

# Section 6 — Reporting Cadence

## Weekly (Quality Lead)

Open repair actions by cell — count and age

New actions raised this week by cell

Actions closed this week

Any action open more than 30 days: flagged for escalation

## Monthly (Management Review)

Total repair actions vs prior month vs same month last year

Defect category breakdown — top 10 and trend

Cell-level breakdown — who generated, who closed

Customer-initiated defects — count and description

Root cause closure rate (target: 100%)

Repair time hours by cell (cost of poor quality)

Top 5 recurring issues with corrective action status

Jobs with highest repair action count — retrospective on outliers

## Quarterly (Leadership)

Trend analysis — is the quality improvement programme working?

Recurring issues resolved vs still open

BOM and checklist improvements completed

Customer defect trend

KPI scorecard update

# Section 7 — Immediate Actions (Next 30 Days)

| Action | Owner | Due | Priority |
| --- | --- | --- | --- |
| Add recurring items to standard BOMs (spare wheel, ladder, nutserts etc.) | Engineering | Week 1 | Critical |
| Make Initiator field mandatory at creation | Quality / IT | Week 1 | Critical |
| Make Root Cause Owner mandatory before closure | Quality / IT | Week 1 | Critical |
| Add Repair Time field — mandatory at closure | Quality / IT | Week 2 | High |
| Retire "General" label — implement cell-specific labels | Quality / IT | Week 2 | High |
| Add Concern Category dropdown (retire free text) | Quality / IT | Week 3 | High |
| Add Root Cause Category dropdown | Quality / IT | Week 3 | High |
| Audit 108 items open more than 180 days | Quality Lead | Week 4 | High |
| Review 539 Won't Do / Hold / Duplicate items | Quality Lead | Week 4 | Medium |
| Run retrospective on top 5 jobs by repair action count | Quality + Production | Week 4 | Medium |
| Assign internal data analysis owner | Management | Week 2 | High |
| Set monthly volume target (baseline: 195, June 2026) | Quality + Management | Week 2 | High |

# Section 8 — Data Ownership

Minimum viable (start now): An internal person with Excel/Power BI capability at 20–30% of their time. Monthly report, trend flags, corrective action register.

Longer term: A dedicated Quality/Continuous Improvement role. At current repair action volume and growth trajectory, this role pays for itself in rework hours saved.

Critical requirement: The data owner needs authority to require corrective action, not just report on it. Reporting without authority produces dashboards, not improvement.